SMARTGRID-DQN

Complete Project Documentation

DQN-Based Battery Energy Management for Renewable Microgrids

S. Thanuj | B.E. EEE | SSN College of Engineering

# 1. Project Overview

SMARTGRID-DQN is a Deep Q-Network (DQN) based energy management system for a renewable microgrid. The system learns when to charge and discharge a battery, and when to import from the grid — to minimize electricity cost and maximize use of solar energy.

The project has two major parts:

- MATLAB Simulink — physical model of the microgrid (PV, Battery, Load, Grid)

- Python DQN — reinforcement learning agent that manages the battery

One line summary:

"A DQN agent that learns to manage a battery in a solar microgrid — minimizing grid dependency over a 24 hour cycle — validated against a Simulink physical model."

| Component | Technology | Purpose |
| --- | --- | --- |
| Microgrid Physical Model | MATLAB Simulink | Simulate PV, Battery, Load, Grid |
| Environment | Python (microgrid_env.py) | State, action, reward interface |
| DQN Agent | Python + PyTorch | Learn optimal battery dispatch |
| Baseline | Python (baseline.py) | Rule-based benchmark to beat |
| Validation | evaluate.py | Compare DQN vs Baseline |

# 2. MATLAB Simulink Model

The Simulink model is the physical foundation of the project. It simulates how power flows between solar panels, battery, loads, and the grid over a 24 hour period.

## 2.1 Overall Architecture

The model has 4 subsystems connected through a central Power Balance block:

| Subsystem | Input | Output | EE Concept |
| --- | --- | --- | --- |
| PV Subsystem | Simulation time | P_pv (kW) | Irradiance to power conversion |
| Load Subsystem | Simulation time | P_load (kW) | Residential demand profile |
| Battery Subsystem | P_cmd | SoC | Energy storage dynamics |
| Grid Subsystem | P_deficit | P_grid (kW) | Backup power import |
| Power Balance | P_pv, P_load, SoC | P_cmd, P_deficit | KCL at DC bus |

## 2.2 PV Subsystem

Simulates solar panel output across 24 hours using a sine wave to approximate the sun's position.

Blocks inside:

- Sine Wave (IRRADIANCE) — Amplitude: 1, Frequency: pi/43200, Phase: -pi/2, Sample time: 3600

- Gain (O/P Power) — Value: 9, scales irradiance to ~9 kW peak power

- Saturation (only positive power) — clips negative sine values to 0 (no negative solar)

- Outport — P_pv signal out

Key formula:

P_pv = 9 x max(0, sin((pi/43200) x t - pi/2))

This gives a bell curve peaking at hour 12 (noon) and zero at night.

## 2.3 Battery Subsystem

Models the State of Charge (SoC) dynamics of a 50 kWh lithium battery. Uses a Unit Delay + MATLAB Function instead of a continuous integrator — this is physically correct because real BMS systems use previous SoC to make current decisions.

Blocks inside:

- Inport — P_cmd (charge/discharge command in kW)

- Unit Delay — holds previous SoC, initial condition 0.5, sample time 3600s

- MATLAB Function (SoC_update) — updates SoC each hour

- Outport — SoC signal out

SoC update logic:

SoC_new = SoC_prev + P_cmd / 50

SoC_new = clamp(SoC_new, 0.2, 0.9)

Why Unit Delay instead of Integrator?

The Integrator caused an algebraic loop error — Simulink couldn't solve SoC and P_cmd simultaneously because each depends on the other. Unit Delay breaks this by using SoC from the previous timestep. This is physically correct — in real systems you can't know current SoC before deciding the command.

## 2.4 Load Subsystem

Simulates residential electricity demand over 24 hours — low at night, peaks in the evening.

Blocks inside:

- Sine Wave (Baseload) — Amplitude: 2, Bias: 3, Phase: pi (peaks opposite to PV)

- Constant (Constant_load) — Value: 1 (always-on loads: fridge, router etc.)

- Sum (total load) — adds sine + constant

- Saturation (load limits) — minimum 0.5 kW, never goes to zero

- Outport — P_load signal out

## 2.5 Grid Subsystem

Import-only grid connection. Acts as backup when PV + Battery cannot meet the load.

Blocks inside:

- Inport — P_deficit (unmet load from Power Balance)

- Saturation (Grid Limits) — Lower: 0 (no export), Upper: 10 kW (max import)

- Gain — Value: 1 (pass-through, tariff logic can be added here later)

- Outport — P_grid signal out

## 2.6 Power Balance Block

The brain of the Simulink model. Receives PV output, load demand, and battery SoC — decides the battery command and calculates unmet load for grid.

MATLAB Function logic:

P_net = P_pv - P_load

if P_net > 0 and SoC < 0.9  =>  charge battery (min of surplus or 5 kW)

if P_net < 0 and SoC > 0.2  =>  discharge battery (max of deficit or -5 kW)

else                         =>  idle, P_deficit = max(0, -P_net)

The Unit Delay on the SoC feedback wire (main canvas) breaks the algebraic loop between Power Balance and Battery Subsystem.

## 2.7 Simulation Settings

| Setting | Value | Reason |
| --- | --- | --- |
| Stop Time | 86400 seconds | 24 hours in seconds |
| Fixed Step Size | 3600 seconds | One decision per hour |
| Solver | ode3 (Fixed-step) | Stable for discrete energy systems |
| Battery Capacity | 50 kWh | Realistic residential/small commercial |
| Max Charge/Discharge | 5 kW | C-rate limit |
| SoC Range | 0.2 to 0.9 | Protects battery life |

## 2.8 Data Export

After simulation, data is exported to MATLAB workspace and saved as a .mat file for Python:

PV_data   = out.PV_data.Data;

Load_data = out.Load_data.Data;

SoC_data  = out.Soc_data.Data;

Grid_data = out.P_grid.Data;

save('microgrid_data.mat', 'PV_data', 'Load_data', 'SoC_data', 'Grid_data', 'time')

# 3. Reinforcement Learning & DQN — From Basics

## 3.1 What is Reinforcement Learning?

Reinforcement Learning (RL) is a method where an agent learns by trial and error. It takes actions in an environment, receives feedback (reward), and learns to make better decisions over time.

In your microgrid:

| RL Term | Microgrid Meaning |
| --- | --- |
| Agent | The battery controller (DQN) |
| Environment | The microgrid (PV + Battery + Load + Grid) |
| State | Current SoC, PV power, Load demand, Hour of day |
| Action | Charge battery / Discharge battery / Idle |
| Reward | Negative grid import + SoC violation penalty |
| Episode | One complete 24-hour day (24 timesteps) |

## 3.2 What is Q-Learning?

Q-Learning assigns a quality score Q(state, action) to every possible state-action pair. The agent always picks the action with the highest Q value.

Example at midnight with low battery:

State: [SoC=0.3, PV=0, Load=2, hour=23]

Q(state, idle)      = -2.1

Q(state, charge)    = -8.5   (no solar at midnight, bad idea)

Q(state, discharge) = -1.3   <-- highest, agent picks this

The Bellman Equation updates Q values during training:

Q(state, action) = reward + gamma x max Q(next_state, all_actions)

This means: the quality of an action = immediate reward + best possible future reward. Gamma (0.99) controls how much future rewards matter vs immediate ones.

## 3.3 Why Neural Network? (The Deep in DQN)

Calculating Q for every possible state manually is impossible — SoC alone has infinite continuous values. So a neural network approximates the Q function:

Input:  [SoC, P_pv, P_load, hour]  -- 4 values

Layer 1: 64 neurons + ReLU

Layer 2: 64 neurons + ReLU

Output: [Q_idle, Q_charge, Q_discharge]  -- 3 values

Training adjusts the network weights so Q predictions get closer to the Bellman target.

## 3.4 Key DQN Components

| Component | What it does | Why it matters |
| --- | --- | --- |
| Experience Replay | Stores past (state, action, reward, next_state) in memory, trains on random batches | Breaks correlation between sequential steps, stable learning |
| Epsilon-Greedy | Acts randomly with probability epsilon, uses network otherwise | Balances exploration vs exploitation |
| Target Network | Frozen copy of policy network, updated every 10 episodes | Provides stable learning target, prevents oscillation |
| Bellman Update | target_q = reward + gamma x max_next_q | Core update rule — makes Q predictions accurate |

## 3.5 Epsilon Decay

Epsilon controls how often the agent explores randomly vs follows learned policy:

Episode 1:    epsilon = 1.00  -- 100% random (knows nothing)

Episode 200:  epsilon = 0.09  -- 9% random

Episode 400+: epsilon = 0.01  -- 1% random (mostly learned policy)

Every episode: epsilon = epsilon x 0.9995

# 4. Python Files — Complete Explanation

## 4.1 config.py

Central settings file. Every hyperparameter lives here — change once, reflects everywhere. No hardcoded numbers scattered across files.

| Parameter | Value | Meaning |
| --- | --- | --- |
| BATTERY_CAPACITY | 50 kWh | Total battery size |
| SOC_MIN / SOC_MAX | 0.2 / 0.9 | Operating range to protect battery life |
| STATE_SIZE | 4 | SoC, P_pv, P_load, hour |
| ACTION_SIZE | 3 | idle, charge, discharge |
| LEARNING_RATE | 0.001 | How fast network weights update |
| GAMMA | 0.99 | Future reward discount factor |
| EPSILON_START | 1.0 | Start fully random |
| EPSILON_DECAY | 0.9995 | Slow decay for thorough exploration |
| MEMORY_SIZE | 10000 | Replay buffer capacity |
| BATCH_SIZE | 64 | Samples per training step |
| NUM_EPISODES | 2000 | Total training days |
| TARGET_UPDATE_FREQ | 10 | Update target network every N episodes |

## 4.2 microgrid_env.py

The environment — simulates the physical microgrid in Python. Works like a gym: give it an action, get back next state and reward. Loads real PV and load profiles from Simulink .mat file.

Key methods:

- reset() — resets hour to 0, SoC to 0.5, returns initial state

- step(action) — executes action, updates SoC, calculates grid import, returns (next_state, reward, done, info)

- _get_state() — returns [SoC, P_pv, P_load, hour/24]

- _get_reward() — grid import penalty (-2 per kW) + SoC violation penalty (-10 at min, -2 at max)

Action to power command logic:

action 0 (idle):      P_cmd = 0

action 1 (charge):    P_cmd = min(MAX_CHARGE, max(0, P_net))

action 2 (discharge): P_cmd = -min(MAX_DISCHARGE, max(0, -P_net))

## 4.3 utils.py

Helper functions for visualization and logging. Called by both baseline and training.

- plot_results() — 3 subplots: episode rewards, SoC over 24hrs, grid import per hour

- print_episode_stats() — formatted console output every 50 episodes

- moving_average() — smooths noisy reward curves using convolution

## 4.4 baseline.py

Rule-based controller — the benchmark DQN must beat. Uses simple if-else logic on the same environment.

Rule-based policy:

if P_net > 0 and SoC < 0.9:  action = charge

elif P_net < 0 and SoC > 0.2: action = discharge

else:                          action = idle

Run this first before training — it tells you the baseline reward and grid import to beat.

## 4.5 train_rl.py

Main training file. Contains two classes and a training loop:

DQN class (Neural Network):

- 3 fully connected layers: 4 -> 64 -> 64 -> 3

- ReLU activations between layers

- Input: state vector, Output: Q values for each action

DQNAgent class:

- policy_net — trained every step

- target_net — frozen copy, updated every 10 episodes

- select_action() — epsilon-greedy action selection

- store() — saves experience to replay buffer

- train_step() — samples batch, computes Bellman target, backpropagates loss

Training loop:

- 2000 episodes, each episode = 24 hour day

- Every step: select action, execute, store experience, train

- Every 10 episodes: update target network

- Best model saved to dqn_model.pth

- Stats printed every 50 episodes

## 4.6 evaluate.py

Loads the saved best model and runs a head-to-head comparison against the baseline on the same environment.

- run_dqn() — loads dqn_model.pth, epsilon=0 (pure exploitation), runs 24hr episode

- run_baseline() — runs rule-based policy for 24hr episode

- plot_comparison() — side by side SoC curves and grid import bar charts

# 5. Results

## 5.1 Training Progress

| Episode | Reward | Grid Import | Observation |
| --- | --- | --- | --- |
| 0 | -66.58 | 33.29 kWh | Fully random, terrible performance |
| 50 | -61.72 | 15.86 kWh | Starting to learn, epsilon still high |
| 100 | -20.31 | 10.15 kWh | Significant improvement |
| 150 | -13.31 | 4.65 kWh | Near-optimal policy found |
| 400+ | -9.31 | 4.65 kWh | Converged and stable |

## 5.2 DQN vs Baseline Comparison

| Metric | Rule-Based Baseline | DQN Agent | Improvement |
| --- | --- | --- | --- |
| Total Reward | -17.31 | -9.31 | 46% better |
| Grid Import | 4.65 kWh | 4.65 kWh | Same grid usage |
| SoC Management | Drops to 0.26 | Stays above 0.25 | Better SoC control |
| Strategy | Reactive rule-based | Learned predictive policy | Forward-looking |

Key insight:

DQN achieved 46% better reward with the same grid import — by learning smarter SoC management that avoids boundary violations. The rule-based agent drains battery close to minimum SoC, triggering penalties. DQN learned to keep SoC in a safer range.

# 6. Complete Project Flow

| Step | Tool | What happens |
| --- | --- | --- |
| 1. Build PV subsystem | Simulink | Sine wave -> Gain -> Saturation -> P_pv output |
| 2. Build Load subsystem | Simulink | Sine wave + Constant -> Sum -> P_load output |
| 3. Build Battery subsystem | Simulink | P_cmd -> MATLAB Fn (SoC update) -> SoC output |
| 4. Build Grid subsystem | Simulink | P_deficit -> Saturation -> P_grid output |
| 5. Build Power Balance | Simulink | KCL node: computes P_cmd and P_deficit |
| 6. Close feedback loop | Simulink | Unit Delay on SoC wire breaks algebraic loop |
| 7. Run and validate | Simulink | Verify SoC profile matches expected behavior |
| 8. Export data | MATLAB | Save PV, Load, SoC, Grid to microgrid_data.mat |
| 9. Load in Python | Python | scipy.io.loadmat reads Simulink profiles |
| 10. Run baseline | Python | Rule-based agent sets benchmark: -17.31 reward |
| 11. Train DQN | Python | 2000 episodes, agent learns optimal policy |
| 12. Evaluate | Python | DQN beats baseline: -9.31 reward, 46% improvement |

# 7. How to Explain This Project

## One line (for anyone):

"I built an AI that manages a solar battery system — it learns when to store energy and when to use it, reducing dependence on the grid."

## To an EE professor:

"I developed a DQN-based energy management system for a grid-connected PV-battery microgrid. The physical plant was modeled in MATLAB Simulink with a 50 kWh battery, 9 kW PV array, and residential load profile. The DQN agent was trained on Simulink-generated data using experience replay and a target network, achieving 46% reward improvement over a rule-based baseline with equivalent grid import reduction."

## If asked about Simulink:

"The Simulink model has four subsystems — PV, Battery, Load, and Grid — connected through a Power Balance block that implements KCL at the DC bus. I used a Unit Delay instead of a continuous integrator for the battery SoC to avoid algebraic loop errors, which is also physically accurate since real BMS systems use sampled SoC data."

## If asked about the DQN:

"The DQN has a 3-layer fully connected network taking 4 state inputs — SoC, PV power, load, and hour — and outputting Q values for 3 actions: charge, discharge, or idle. I used experience replay with a 10,000 sample buffer, epsilon-greedy exploration with decay from 1.0 to 0.01 over 2000 episodes, and a target network updated every 10 episodes for training stability."

## If asked why DQN over rule-based:

"The rule-based controller is reactive — it charges when there is surplus and discharges when there is deficit. The DQN is predictive — it learns to account for time of day, anticipate future load patterns, and avoid draining the battery to critical levels. This showed up as 46% better reward in our evaluation."

# 8. Project File Structure

| File | Type | Purpose |
| --- | --- | --- |
| microgrid_env.slx | Simulink | Physical microgrid model |
| microgrid_data.mat | MATLAB data | Exported simulation profiles |
| config.py | Python | All hyperparameters |
| microgrid_env.py | Python | RL environment (gym-style) |
| utils.py | Python | Plotting and logging helpers |
| baseline.py | Python | Rule-based benchmark |
| train_rl.py | Python | DQN agent + training loop |
| evaluate.py | Python | DQN vs baseline comparison |
| dqn_model.pth | PyTorch model | Saved best trained model |
| results/comparison.png | Image | Final comparison plot |

End of Documentation